HOME AUTOMATION USING NODEMCU(ESP8266) IOT( Online Network Interfacing )

A PROJECT REPORT

Submitted by

Tumun shaily

ABSTRACT

This project presents a design and prototype implementation of new home automation system that uses Cloud technology as a network infrastructure connecting its parts. The proposed system consists of two main components; the first part is the server (web server), which presents system core that manages, controls, and monitors users’ home.

Users and system administrator can locally (LAN) or remotely (internet) manage and control system code. Second part is hardware interface module, which provides appropriate interface to sensors and actuator of home automation system.

Unlike most of available home automation system in the market the proposed system is scalable that one server can manage many hardware interface modules

as long as it exists on WiFi network coverage. System supports a wide range of home automation devices like power management components, and security components.

The proposed system is better from the scalability and flexibility point of view than the commercially available home automation systems.

TABLE OF CONTENTS

Contents

ABSTRACT………………………………………………………………………

CHAPTER NO.01………………………………………………………………..

INTRODUCTION………………………………………………………………...

CHAPTER NO.02……………………………………………………………….

OBJECTIVE OF THE PROJECT………………………………………………...

CHAPTER NO.03…………………………………………………………………

LITERATURE SURVEY……..…………………………………………………..

CHAPTER NO.04…………………………………………………………………

SCOPE OF THE PROJECT…………………………………………..…………

CHAPTER NO.05…………………………………………………………………

METHODOLOGY …………...………………………………………...…….….

CHAPTER NO.06…………………………………………………………………

HARDWARE…………………………………………………………………….

CHAPTER NO.07…………………………………………………………………

SOFTWARE….……………………….….……………………………………...

CHAPTER NO.08……………………………………………………………….

REFRENCES & BIBLIOGRAPHY……………………………..………………

CHAPTER NO.01

INTRODUCTION

INTRODUCTION

The project aims at designing an advanced home automation system using Blynk web server and Nodemcu technology. The devices can be switched ON/OFF and sensors can be read using a Voice commands like google assistant , Alexa , etc, through Wi-Fi.

Automation is the most frequently spelled term in the field of electronics. The hunger for automation brought many revolutions in the existing technologies. These had greater importance than any other technologies due to its user-friendly nature. These can be used as a replacement of the existing switches in home which produces sparks and also results in fire accidents in few situations. Considering the advantages of Wi-Fi an advanced automation system was developed to control the appliances in the house.

Wi-Fi (Short for Wireless Fidelity) is a wireless technology that uses radio frequency to transmit data through the air. Wi-Fi has initial speeds of 1mbps to 2mbps. Wi-Fi transmits data in the frequency band of 2.4 – 2.5 GHz. It implements the concept of frequency division multiplexing technology. Range of Wi-Fi technology is 40-300 feet.

The controlling device for the automation in the project is a Nodemcu (Esp8266). The data sent from Mobile over Wi-Fi will be received by  Nodemcu and

Arduino UNO. Arduino UNO reads the data and decides the webhook switch and process the data send signals to the nodemcu ,then nodemcu decide to turn on/off electrical devices connected to it through Relays.

CHAPTER NO.02

OBJECTIVE OF PROJECT

- The goal of this project is to develop a home automation system that gives the user complete control over all remotely controllable home.

- The automation system will have the ability to be controlled from a central host PC, the Internet, and also remotely accessed via a Mobile phone and Blynk server , IFTTT services and our own application(In future though!).

- Future Objectives

- The System will also sense the Accidental Gas leakage , water level and will notify the user by SMS.

- Can check for water level checks and Fire Alarm (when no one is around).

CHAPTER NO.03

LITERATURE SURVEY

Literature survey:

Review of Related Literature:

When people think about home automation, most of them may imagine living

in a smart home: One remote controller for every household appliance, cooking

the rice automatically To some extent home automation equals to smart home. They both bring out smart living condition and make our life more convenient and fast.

And here in IIIT KOTTAYAM we have made it come true .

CHAPTER NO.04

SCOPE OF PROJECT

Day by day, the field of automation is blooming and these systems are having great impact on human beings. The project which is to be implemented is a home automation using Easy IOT Webserver and WIFI and has very good future development.

In the current we are using Blynk webserver through our smartphones so the home appliances can be controlled using webserver on cloud .

We are using Blynk india Ip for network cloud , And using IFTTT services for voice commands .

CHAPTER NO.05

METHODOLOGY

CHAPTER NO.06

HARDWARE

Technical specifications:-

Microcontroller	ATmega328P

Operating Voltage 5V

| Input | Voltage | (recommended) |  | 7-12V |
| --- | --- | --- | --- | --- |
| Input | Voltage | (limit) | 5V |  |

Digital I/O Pins	D3,D4 (of Nodemcu)

| PWM Digital I/O Pins | 6 |
| --- | --- |

DC Current per I/O Pin	20 mA

DC Current for 3.3V Pin 50 mA

| Flash Memory |  |  | 32 | KB | (ATmega328P) |
| --- | --- | --- | --- | --- | --- |
| of which 0.5 |  | KB used |  | by bootloader |  |
| SRAM | 2 KB (ATmega328P) |  |  |  |  |
| EEPROM |  | 1 KB (ATmega328P) |  |  |  |
| Clock Speed 16 |  |  | MHz |  |  |
| Length |  | 68.6 mm |  |  |  |
| Width |  |  | 53.4 | mm |  |
| Weight |  | 25 | g. |  |  |

Arduino Code:-

#include "SoftwareSerial.h"

#define DEBUG true

SoftwareSerial esp8266(2,3); // make RX Arduino line is pin 2, make TX Arduino line is pin 3.

- This means that you need to connect the TX line from the esp to the Arduino's pin 2

// and the RX line from the esp to the Arduino's pin

3

void setup()

{

Serial.begin(9600);

esp8266.begin(9600); // your esp's baud rate might be different

pinMode(4,OUTPUT);

digitalWrite(4,LOW);

pinMode(5,OUTPUT);

digitalWrite(5,LOW);

pinMode(6,OUTPUT);

digitalWrite(6,LOW);

pinMode(7,OUTPUT);

digitalWrite(7,LOW);

pinMode(13,OUTPUT);

digitalWrite(13,LOW);

sendData("AT+RST\r\n",2000,DEBUG); // reset module sendData("AT+CWMODE=2\r\n",1000,DEBUG); // configure as access point sendData("AT+CIFSR\r\n",1000,DEBUG); // get ip address

sendData("AT+CIPMUX=1\r\n",1000,DEBUG); // configure for multiple connections

sendData("AT+CIPSERVER=1,80\r\n",1000,DEBUG); // turn on server on port 80

}

void loop()

{

if(esp8266.available()) // check if the esp is sending a message

{

if(esp8266.find("+IPD,"))

{

delay(1000); // wait for the serial buffer to fill up (read all the serial data)

- get the connection id so that we can then disconnect

int connectionId = esp8266.read()-48; // subtract 48 because the read() function returns

// the ASCII decimal value and 0

(the first decimal number) starts at 48

esp8266.find("pin="); // advance cursor to "pin="

int pinNumber = (esp8266.read()-48)*10; // get first number i.e. if the pin 13 then the 1st number is 1, then multiply to get 10

pinNumber += (esp8266.read()-48); // get second number, i.e. if the pin number is 13 then the 2nd number is 3, then add to the first number

switch (pinNumber)

{

case 1://switch 1 on

digitalWrite(4,HIGH);

break;

case 2://switch 2 on

digitalWrite(5,HIGH);

break;

case 3://switch 3 on

digitalWrite(6,HIGH);

break;

case 4://switch 4 on

digitalWrite(7,HIGH);

break;

case 5://led on

digitalWrite(13,HIGH);

break;

case 6://switch 1 off

digitalWrite(4,LOW);

break;

case 7://switch 2 off

digitalWrite(5,LOW);

break;

case 8://switch 3 off

digitalWrite(6,LOW);

break;

case 9://switch 4 off

digitalWrite(7,LOW);

break;

case 10://led off

digitalWrite(13,LOW);

break;

default:

break;

}

//digitalWrite(pinNumber, !digitalRead(pinNumber)); // toggle pin

// make close command

String closeCommand = "AT+CIPCLOSE=";

closeCommand+=connectionId; // append connection id

closeCommand+="\r\n";

sendData(closeCommand,1000,DEBUG); // close connection

}

}

}

/*

- Name: sendData

- Description: Function used to send data to ESP8266.

- Params: command - the data/command to send; timeout - the time to wait for a response; debug - print to Serial window?(true = yes, false = no)

- Returns: The response from the esp8266 (if there is a reponse)

*/

String sendData(String command, const int timeout, boolean debug)

{

String response = "";

esp8266.print(command); // send the read character to the esp8266 long int time = millis();

while( (time+timeout) > millis())

{

while(esp8266.available())

{

if(debug)

{

Serial.print(response);

return response;

}

Esp 8266:-

Description: The ESP8266 WiFi Module is a self contained SOC with integrated TCP/IP protocol stack that can give any microcontroller access to your WiFi network. The ESP8266 is capable of either hosting an application or offloading all Wi-Fi networking functions from another application processor. Each ESP8266 module comes pre-programmed with an AT command set firmware, meaning, you can simply hook this up to your Arduino device and get about as much WiFi-ability as a WiFi Shield offers (and that’s just out of the box)! The ESP8266 module is an extremely cost effective board with a huge, and ever growing, community.

This module has a powerful enough on-board processing and storage capability that allows it to be integrated with the sensors and other application specific devices through its GPIOs with minimal development up-front and minimal loading during runtime. Its high degree of on-chip integration allows for minimal external circuitry, including the front-end module, is designed to occupy minimal PCB area. The ESP8266 supports APSD for VoIP applications and Bluetooth co-existance interfaces, it contains a self-calibrated RF allowing it to work under all operating conditions, and requires no external RF parts.

There is an almost limitless fountain of information available for the ESP8266, all of which has been provided by amazing community support. In the Documents section below you will find many resources to aid you in using the ESP8266, even instructions on how to transforming this module into an IoT (Internet of Things) solution!

Features:

- 802.11 b/g/n

- Wi-Fi Direct (P2P), soft-AP

- Integrated TCP/IP protocol stack

- Integrated TR switch, balun, LNA, power amplifier and matching network

- Integrated PLLs, regulators, DCXO and power management units

- +19.5dBm output power in 802.11b mode

- Power down leakage current of <10uA

- 1MB Flash Memory

- Integrated low power 32-bit CPU could be used as application processor

- SDIO 1.1 / 2.0, SPI, UART

- STBC, 1×1 MIMO, 2×1 MIMO

- A-MPDU & A-MSDU aggregation & 0.4ms guard interval

- Wake up and transmit packets in < 2ms

- Standby power consumption of < 1.0mW (DTIM3)

Specification of ESP 8266:

- Wi-Fi Direct (P2P), soft-AP

- Integrated TCP/IP protocol stack

- Integrated TR switch, balun, LNA, power amplifier and matching network

- Integrated PLLs, regulators, DCXO and power management units

- 19.5dBm output power in 802.11b mode

- Power down leakage current of <10uA

- 1MB Flash Memory

- Integrated low power 32-bit CPU could be used as application processor

- Standby power consumption of < 1.0mW (DTIM3)

Relayboard:-

A relay is an electrical device which is generally used to control high voltages using very low voltage as an

Input. This consists of a coil wrapped around a pole and a two small metal flaps(nodes) that are used to close the circuit. One of the node is fixed and other is movable. Whenever an electricity is passed through the coil, it creates a magnetic field and attracts the moving node towards the static node and the circuit gets completed. So, just by applying small voltage to power up the coil we can actually complete the circuit for the high voltage to travel. Also, as the static node is not physically connected to the coil there is very less chance that the Microcontroller powering the coil gets damaged if something goes wrong.

This is Four Channel relay board controlled by computer USB port. The usb relay board is with 4 SPDT relays rated up to 10A each. You may control devices 220V / 120V (up to 4) directly with one such relay unit. It is fully powered by the computer USB port. Suitable for home automation applications, hobby projects, industrial automation. The free software allows to control relays manually, create timers (weekly and calendar) and multivibrators, use date and time for alarms or control from command line. We provide software examples in Labview, .NET, Java, Borland C++, Python

Features:-

- Datasheet - here

- Power led: Yes

- Relay leds: YesHigh quality

- 4 SPDT Relay channels - selectable by user:

35

| o |  | JQC-3FC/T73 DC5V (7A / 250VAC, 10A / 125VAC, 12A / 120VAC, 10A / |  |
| --- | --- | --- | --- |
|  | 28VDC) |  |  |
| o |  | RAS-05-15 (10A / 250VAC, 15A / 120VAC, 15A / 24VDC) |  |

- PCB parameters: FR4 / 1.5mm / two layers / metalized holes / HAL / white

stamp / solder mask / еxtra PCB openings for better voltage isolation / doubled high voltage tracks

- Power supply: from USB port

- Current consumption: 400 mA

- Chip: FT245RL

- Size: 77mm x 56mm x 17mm

- Supported by DRM software (Windows and Linux): Yes

- Supported by Denkovi Command line tool (Windows, Linux): Yes

- Android software available (low cost but very useful): Yes - New

- Software examples - here

- Documentation: here

Advantages:-

- High quality

- Low cost

- No extra power supply

- Software with many functions

- Control electrical devices according weekday/date/time

- Create timers or pulses with our software

Applications:-

- Home automation

- Robotics

- Alarms

- Timers

- Open doors and windows via PC

- Aquariums applications

Additional information:-

This is relay board with 4 SPDT Relays controlled from USB port of your computer. The main purpouse of this USB relay module is to help you building projects regarding robotics and home automation (domotic). You may control differenet electrical devices like home lights, DC motors, pneumatic cylinders, lasers and so on. Each such board requires one USB port. The more USB ports you have the more such relay units you may connect and control. . The relay module outputs are controlled by FT245RL. It has 8 bit data output register (this device use only 4 of them). The usb relay card can not be controlled directly via COM port - you need to download our DRM Software to control the device. The usb relay unit can not work without PC. Only one such device can be supplyed from single USB port. If you want to supply many such devices you need USB HUB with extra power supply.

USB TO TTL CONVERTER:-

This USB to TTL converter combine the USB-232-1 (USB to Single RS232 Adapter) and TTL-232-1 (Port-powered RS232/TTL converter) allows you to convert USB to TTL/CMOS compatible levels and vice versa.

It can be used to set up APC220 Radio Data Module(SKU:TEL0005) wireless module.

It can be used as STC microcontroller program downloader.

Specification

- Voltage: 5V

- Chip: Silicon Laboratories CP210x chip

Pin Definition:

CHAPTER NO.07

SOFTWARE

Esp 8266 firmware:-

Required hardware and software:

|  | You will need a Windows PC for this update |  |  |
| --- | --- | --- | --- |
|  | You will need some form of USB to Serial converter that allows |  |  |
|  | operation at 3.3V. I used a Focaboard. It allows easy plugging into a |  |  |
|  | breadboard, which then allows me to hookup the pins of the ESP8266 module |  |  |
|  | via jumper wires to the corresponding pins on the USB<->Serial board. |  |  |
|  | The firmware updating software only works on COM ports 1-6. If |  |  |
|  | your USB<->Serial device enumerates to a higher port number than that, you |  |  |
|  | will have to change it via Device Manager in Windows |  |  |

Above is a pin out diagram for the ESP8266 Module

You need to hookup these pins from the ESP8266 to your USB<->Serial board:

- VCC to 3.3V

- GND to ground

- CH_PD to 3.3V

- TXD to RX, RXD to TX (this may depend on the USB<->Serial board you are using. If it doesn't work, try swapping them around)

- GPIO0 to ground (for the duration of firmware upgrading. After all the upgrades have been loaded, it needs to be disconnected)

You will need to unplug and re-plug the USB cable 4 times during the process, so make sure you can reach it easily

Flashing the firmware

Inside the esp_flasher.zip file, you will find a couple of .bin files, an executable named XTCOM_UTIL.exe and also a readme.txt file containing the addresses to flash each of the .bin files to, which I will duplicate here for reference

boot_v1.1.bin---------------->0x00000

user1.bin-------------------->0x01000

esp_init_data_default.bin---->0x7C000

blank.bin-------------------->0x7E000

Steps to flash each .bin file

1.  Power everything by plugging into USB port.2.Start XTCOM_UTIL.exe

3.Hit Tools -> Config Device in the menu

- Configure the settings to whatever COM port you are using, and 9600 baud rate

- Click on "Open". You should receive a notification saying "Operation Succeeded!"

- Click on "Connect". The "Try to connect times: " message should increment a few times as the utility tries to contact the module.

You should receive a notification about success before hitting 20 times.If you don't, then make sure your connections are correct, and perhaps try swapping the RX and TX pins around.

- Close the "Config Device" window.

- Hit API TEST -> (4) Flash Image Download in the menu

- Select the .bin file to flash, and configure the correct destination address as in the readme.txt file (For Example: boot_v1.1.bin @ 0x00000)

- Click on "Download" and wait for the flashing to complete.

- Close down the entire XTCOM_UTIL program, and unplug the USB. * VERY IMPORTANT This must be done between each .bin file

- Repeat each of the above steps for each of the .bin files, making sure to leave the GPIO0 pin grounded during the entire process.

CHAPTER NO.08

REFERENCE AND BIBLIOGRAPHY

References & Bibliography:-

Websites:

- http://www.iot-playground.com

- http://www.instructables.com

- http://en.wikipedia.org

- http://www.journals.elsevier.com/easyiot

Journals & other books:

- 1 .Kusuma S M, Assistant Professor, Department of telecommunication, MSRIT, Bangalore, India. “Home Automation Using Internet of Things.”

- 2.Niharika Shrotriya, Anjali Kulkarni, Priti Gadhave, International Journal of Science, Engineering and Technology Research (IJSETR), “SMART HOME USING WI-FI”

- Anushri Aware, SonaliVaidya,PriyankaAshture, VarshaGaiwal PES’s Modern College of Engineering, Pune-04, International Journal of Engineering Research and General Science Volume 3, “Home Automation using Cloud Network”